REFERENCE · APPENDIX B EXP → V5 Cheat Sheet Pull this out when you join the team

Everything you learned on EXP, in the words the V5 team uses. A few names change; the thinking does not. A VEX match = an autonomous period + a driver period — and you already do both (your Sumo auto-hunt and your tank driving).

| Concept | A&R · VEX EXP (VEXcode) | Team · VEX V5 (SpartanDrive) |
| --- | --- | --- |
| Robot's computer | EXP Brain — built-in inertial sensor | V5 Brain — built-in IMU |
| The motors | Smart Motor — 200 RPM, fixed | Smart Motor — swappable cartridges |
| Bundle a side's motors | motor_group leftSide = motor_group(...) | the left/right groups SpartanDrive uses |
| Whole-robot drive object | a (smart) drivetrain | the drive object |
| Tank (two-stick) driving | left & right motor_group from the sticks | drive.tank(leftY, rightY) |
| Arcade (one-stick) driving | Drivetrain.arcade(power, turn) | drive.split_arcade(leftY, rightX) |
| Drive a set distance | Drivetrain.driveFor(forward, 24, inches) | drive.drive_distance(24) |
| Turn to a heading (absolute) | Drivetrain.turnToHeading(90, degrees) | drive.turn_to_angle(90) |
| Turn a set amount (relative) | Drivetrain.turnFor(right, 90, degrees) | drive.turn_to_angle(heading + 90) |
| Which way am I facing? | Brain inertial → heading | drive.heading() |
| Where am I on the field? | — not tracked on EXP — | odometry → drive.get_pose() |
| Ready the sensor | inertial calibrates at Brain startup | drive.calibrate() |
| Run-by-itself routine | your Sumo auto-hunt | autonomous() |
| Driver loop (1 drive call/pass) | while (true) { Drivetrain.arcade(...) } | opcontrol() { ... drive.tank(...) } |
| Where you write & run code | VEXcode (VS Code) → Download & Run | PROS — different editor & layout |
| The C++ language & logic | same thinking | same thinking |

On the team — going further (SpartanDrive)

| On the team you'll also meet | What it does |
| --- | --- |
| drive.swing_turn(90, true) | Pivot on one side; the other holds — a wide, sweeping turn. |
| drive.drive_distance_chain(24, 3) | Drive, but exit early (still rolling) to flow into the next move. |
| drive.turn_to_angle_chain(90, 5) | Turn, but exit early to chain into the next move. |
| drive.get_pose() · drive.set_pose(...) | Read or set the robot's (x, y) field position. |
| drive.reset_odom() | Zero the odometry position. |
| drive.drive_raw(left, right) | Raw power to each side — the escape hatch. |
| drive.set_drive_constants(...) · set_turn_constants(...) | Tune the drive / turn PID gains. |
| drive.stop() · drive.hottest_motor_temp() | Stop the drive · check the hottest motor. |

The one line: same C++ thinking, same robot concepts — a handful of methods get renamed and the editor changes when you reach the team. You're not starting over; you're picking up where this course leaves off.

Automation & Robotics · VEX EXP — Student cheat sheet