PROJECT 3.4 · UNIT 3 — PROGRAMMING & AUTOMATION Automation Through Programming Student Edition — VEX EXP · C++ in VS Code

Name: ______________________________     Date: ____________     Period: ________

Introduction

Time to make a robot run on its own. You'll add the two pieces that turn a machine into an automatic one: reading a sensor for input, and using if / else to decide from what it reads. Wrap that in a while loop and the robot senses, decides, and acts over and over, with no driver — the sense → decide → act loop from Chapter 8, in code. Then you'll design, build, and program a real automatic device as a team. Read Chapter 11 in the textbook for the full explanation.

Learning Objectives

- Explain open- vs. closed-loop systems and identify automation as a closed loop (sense → decide → act).

- Read a sensor in code (the Distance Sensor's objectDistance, the Bumper Switch's pressing()) and use it as a condition.

- Write if / else and a while (true) loop so a motor responds to a sensor.

- Design, build, and program an automatic device from the kit, using the engineering design process and teamwork.

- Store a sensor reading in a variable, read a Potentiometer V2's angle and calibrate it against a captured baseline, and use a for loop to average readings.

Key Ideas (quick reference)

- Automation is a closed loop:  sense (read a sensor) → decide (if / else) → act (move a motor), repeated with a while (true) loop. It is the Chapter 8 feedback loop, written in code.

- Reading a sensor:  name it in your device list, then ask for its reading. distance.objectDistance(mm) returns a number like 340 (Smart Port); bumper.pressing() returns true or false (3-Wire port). A reading is just a value your program can check.

- Deciding with if / else:  a condition (a true/false question) goes in the parentheses; one block runs if true, another (with else) if false. Use comparisons <, >, == ; chain else if for more than two cases; curly braces mark each block.

- Repeating with while (true):  an if checks once; a real device must check continuously, so while (true) repeats the block forever. Always put wait(20, msec) at the end of the loop, and make sure something inside it can change.

- Variables, the potentiometer & calibration:  a variable is a named box that stores a value (int, double, bool). The Potentiometer V2 reads a shaft angle (0 to ~330°) on a 3-Wire port, but its zero depends on mounting — so calibrate: capture the home reading into a baseline variable, then use raw - baseline. A for loop repeats a set number of times (e.g., average 20 readings for a steady baseline); while (true) repeats forever.

The Sense–Decide–Act Loop

Read a sensor, decide with if/else, move a motor — and a while loop repeats it forever so the robot keeps responding. This is the Chapter 8 feedback loop, in code.

The Automation Pattern

Name a sensor in your device list, just like a motor:

Two sensors in the device list

// name a sensor just like a motor: motor driveMotor = motor(PORT1); distance frontEye = distance(PORT2); // Smart Port bumper startButton = bumper(Brain.ThreeWirePort.A); // 3-Wire port

Then sense, decide, and act inside a forever loop. This program drives forward but stops itself before hitting something:

A complete automatic behavior

int main() { vexcodeInit(); driveMotor.setVelocity(50, percent); while (true) { // keep checking, forever if (frontEye.objectDistance(mm) < 150) { // something is close driveMotor.stop(); // -> stop } else { driveMotor.spin(forward); // -> keep driving } wait(20, msec); // pause, then check again } }

Read it as the diagram: sense (read frontEye), decide (the if), act (stop or spin), then loop back. The condition is the part in parentheses — a comparison like < 150, or a yes/no reading like startButton.pressing(). Swap the sensor and you get a different machine.

Reading an Angle: Variables, the Potentiometer & Calibration

The bumper gives a yes/no and the Distance Sensor gives millimeters. To know how far a shaft has turned — an arm, a dial — you need a new sensor and one new tool: a place to store a value.

Variables.  A variable is a named box that holds a value you can read, use in math, and change. Give it a type and a name; update it (count = count + 1) or read it in an if. Variables let you save a reading and reuse it, keep a running total, or remember something between loop passes.

Three variables

int count = 0; // a whole number double home = 0; // a number w/ decimals bool running = false; // true or false

The Potentiometer V2.  It measures the angle of the shaft it is mounted on, 0 to about 330°, through a lettered 3-Wire port. You read it as a decimal angle, so store it in a double. Unlike the bumper (pressed or not) or the Distance Sensor (mm to an object), it reports a rotation — perfect for an arm or dial — and it keeps its reading after a restart because the value follows the shaft.

Name it in the device list, then read it

// in the device list, above main: potV2 armPot = potV2(Brain.ThreeWirePort.A); // later, read the angle into a double: double raw = armPot.angle(degrees); // 0 to ~330

Calibration — where is zero?  The potentiometer's zero is wherever it was mounted (its slots even let you nudge it 90° by hand), so a raw 145° is 145° from the sensor's own arbitrary zero, not from level. To fix it, calibrate: at startup, hold the arm at a known home, read the sensor, and store that reading in a variable — the baseline. Then raw - baseline is the angle from home. Same idea as the inertial's calibrate step (Ch13); here you do it yourself, with a variable.

Capture the raw reading once at the home position (the baseline); then the arm's angle from home = raw - baseline. The sensor's own zero is wherever it was mounted.

The for loop.  One analog reading jitters, so average several for a steady baseline — which needs a loop that runs a fixed number of times: a for loop. It starts a counter, runs while it is below a limit, and steps it each pass. Now you know two kinds of loop: while (true) repeats forever; a for loop repeats a set number of times, then moves on.

Calibrate: average 20 readings at home

double sum = 0; for (int i = 0; i < 20; i = i + 1) { sum = sum + armPot.angle(degrees); wait(5, msec); } double home = sum / 20; // the baseline

Putting it together.  Calibrate once at the top of main(), then read the calibrated angle in the while (true) loop and act — raise an arm until it passes 90° from home, then hold. A classic challenge: hold the arm at three heights, one per controller button. Always subtract the baseline before trusting a potentiometer number, and re-calibrate after remounting.

Raise the arm to 90° from home, then hold

int main() { vexcodeInit(); // calibrate: average 20 readings at home double sum = 0; for (int i = 0; i < 20; i = i + 1) { sum = sum + armPot.angle(degrees); wait(5, msec); } double home = sum / 20; armMotor.setStopping(hold); while (true) { double angle = armPot.angle(degrees) - home; if (angle < 90) { // not high enough yet armMotor.spin(forward); } else { armMotor.stop(); } wait(20, msec); } }

Build Constraints

Build constraints (still in force). (1) Build your device from the classroom PLTW Gateway kit only — its bundled motor and sensors are fair game; the separate competition team's V5 parts are off-limits. (2) The whole device must fit one IKEA Kallax cubby — a 33 × 33 cm (13 × 13 in) opening, about 38 cm (15 in) deep. The 33 × 33 face is the binding limit, so plan your gate, sign, or elevator to pack into that square.

Safety

• An automatic robot moves on its own, sometimes when you don't expect it. Keep hands clear of gates, arms, and gears, and be ready to power down. • Secure the device so a moving part can't pinch fingers or knock things off the bench. • Run a program that drives only after you've unplugged the USB-C cable. • Power the Brain down with the X button when you are done, and store the device in its cubby.

Equipment & Materials

- Your computer with VS Code and the VEX extension, and a USB-C cable

- The PLTW Gateway EXP kit: Brain, charged battery, at least one motor, and a sensor (Distance, Bumper, Optical, or Potentiometer V2)

- Your engineering notebook for sketches, pseudocode, and a daily journal

- This project worksheet (design brief, decision matrix, and reflection)

Procedure

- Define the problem.  Read your task, then fill in the design brief (Worksheet 1): what it must do and every constraint — kit only, fits the cubby, which sensor.

- Sketch and choose.  Each teammate sketches a design. Compare them in the decision matrix (Worksheet 2) against your criteria, and pick the best one to build.

- Build the mechanism.  Construct your device from the kit so it fits the cubby. Mount the motor and the sensor where they belong, and wire them to the Brain.

- Plan, then code.  Write pseudocode for the sense → decide → act loop (Worksheet 3) first. Then translate it to C++: name your devices, set up the while (true) loop, and fill in the if / else.

- Test and refine — a little at a time.  Download, watch it run, and fix one thing at a time. Tune your distance threshold or speed until it behaves the way the task needs.

- Evaluate.  Judge how well it works, keep your daily journal (Worksheet 4), and answer the conclusion questions — including whether your device is an open or a closed loop.

Worksheet 1 — Design Brief

Read your task and define the problem before you build.

| Section | Your answer |
| --- | --- |
| Client / who needs it |  |
| Problem statement — what is the need? |  |
| What it must do (design statement) |  |
| Constraints (kit only, fits cubby, sensor) |  |
| Sensor(s) you'll use |  |

Worksheet 2 — Decision Matrix

List your criteria across the top, then score each idea 1–4 (1 = barely meets it, 4 = best possible). Add across for each idea's Total — the highest is your best solution.

| Idea | Criteria 1 | Criteria 2 | Criteria 3 | Total |
| --- | --- | --- | --- | --- |
| A |  |  |  |  |
| B |  |  |  |  |
| C |  |  |  |  |
| D |  |  |  |  |

Worksheet 3 — Plan the Code

Match your project to a pattern (A–D) from the chapter, then fill in the sense → decide → act loop in plain language before you write C++. To pick your threshold, read the sensor value on the Brain's Devices screen at the real trigger distance.

Pseudocode

/* while (true) { read the sensor: ______________________________ if ( ____________________________________ ) { ____________________________________ } else { ____________________________________ } } */

Worksheet 4 — Daily Journal

A couple of sentences per day — especially what got in your way and how you fixed it.

| Day | What we did / obstacles we hit and how we solved them |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |

How You'll Be Assessed

• It works as an automatic, closed-loop device — senses and responds with no one driving it. • The code is a clear sense → decide → act loop — a sensor read, an if/else, a motor — inside while (true) with a wait. • The design process is documented — brief, decision matrix, pseudocode, and a daily journal. • You can point to the exact line that senses, the line that decides, and the line that acts in your own program.

Conclusion Questions

Answer in complete sentences.

- What sensor(s) did you use to solve this problem, and why those?

- Was your device an open-loop or a closed-loop system? Why?

- Describe two malfunctions your team had to troubleshoot, and how you overcame them.

- Describe two responsibilities you held during this project.

- Why does an automatic device need a while (true) loop instead of a single if?

- Write the sense–decide–act idea as pseudocode for this device: "a night-light motor turns a lamp on when the room is dark." Name the sensor reading, the decision, and the action.

- A Potentiometer V2 on a robot arm reads 160 when the arm is parked at its home position. Why can't you treat that as "160°"? What do you store in a variable to fix it, and how do you then find the arm's angle from home? Where would a for loop help?

Automation & Robotics · VEX EXP · C++ in VS Code — Student Edition