ACTIVITY 2.1 · UNIT 2 — MECHANISMS & MOTION Observing Mechanisms Student Edition — VEX EXP · C++ in VS Code

Name: ______________________________     Date: ____________     Period: ________

Introduction

A mechanism is a device that takes one kind of motion or force and changes it into another — its direction, speed, torque, or type. Motion comes in four types: rotary (turning), linear (straight line), reciprocating (back-and-forth in a line), and oscillating (swinging in an arc), and the six simple machines combine into the mechanisms all around us. The mechanism you'll use most this semester is the gear train — and on your EXP robot, whose motor speed is fixed, the gear ratio is the only speed-and-strength dial you have. In this activity you'll observe mechanisms on everyday objects, then build a gear train and feel the speed–torque trade with your own hands. Read Chapter 3 in the textbook for the full explanation.

Learning Objectives

- Classify motion as rotary, linear, reciprocating, or oscillating, and spot the six simple machines inside everyday mechanisms.

- Identify the drive and driven gears in a gear train and explain why meshed gears turn in opposite directions.

- Calculate a gear ratio from tooth counts and predict what happens to speed and torque.

- Choose and justify a gear ratio for an EXP mechanism (drivetrain vs. arm) using the speed–torque trade.

Key Ideas (quick reference)

- Four motions:  rotary (wheel), linear (drawer), reciprocating (saw blade), oscillating (swing). Mechanisms convert one into another — a rack and pinion turns rotary into linear.

- Drive vs. driven:  the drive gear is the one the motor turns (input); the driven gear is the output. Meshed gears always turn in opposite directions. Find the drive gear FIRST — it's where most wrong answers come from.

- Gear ratio = driven teeth : drive teeth.  24T driving 60T → 60:24 = 2.5:1 — the motor turns 2½ times per output turn. Bigger than 1 = slower and stronger; less than 1 = faster and weaker.

- The trade:  torque up means speed down, always. More torque: smaller drive gear or bigger driven gear. Memory hook: slow down to power up.

- Your EXP numbers:  the Smart Motor (5.5W) is fixed at 200 RPM and 0.5 N·m — no swappable cartridges (that's V5). Your kit's gears are 24T, 36T, 48T, and 60T; external gearing is your only ratio tool. 24T→60T gives an arm 2.5× the torque at 80 RPM.

Equipment & Materials

- VEX EXP parts: 24T/36T/48T/60T gears, shafts, bearing flats, shaft collars, flat bars or C-channels

- Everyday objects with mechanisms (scissors, eggbeater, can opener, bicycle)

- Engineering notebook

Procedure

- See the four motions.  Watch the demonstration of rotary, linear, reciprocating, and oscillating motion; copy the four into your notebook with one example of your own per type.

- Observation stations.  At each station, record the input motion, the mechanism (and the simple machines inside it), and the output motion in the table below.

- Build and feel the trade.  Mount a 24T gear meshed with a 60T gear on a flat bar (bearing flats behind each shaft, collars to hold them). Turn the 24T by hand while your partner gently resists the 60T shaft — feel how strong the slow shaft is. Then drive the 60T and feel the 24T spin fast but give up easily. Write one sentence about what you felt.

- Complete the gear-ratio practice table  below — the first row is worked for you.

- Class discussion:  which simple machines appeared at each station, and where did motion change type?

Observation Stations

| Station / object | Input motion | Mechanism & simple machines | Output motion |
| --- | --- | --- | --- |
| Eggbeater (example) | Rotary (hand crank) | Gear train — wheel & axle + gears; big drive gear, small driven gears → speeds up | Rotary (faster) |

Gear-Ratio Practice Table

Ratio = driven teeth : drive teeth. Bigger than 1 → slower and stronger. The motor spins 200 RPM.

| Drive (input) teeth | Driven (output) teeth | Gear ratio | What happens to speed & torque? |
| --- | --- | --- | --- |
| 24 | 48 | 48:24 = 2:1 | Drive turns twice per output turn — speed halves (200 → 100 RPM), torque doubles. |
| 36 | 60 |  |  |
| 48 | 48 |  |  |
| 60 | 24 |  |  |
| 24 | 60 |  |  |

Engineering Drawing

The 24T and 60T gears as a technical drawing. Because pitch Ø = teeth ÷ 24, the center distance is just the average of the two pitch radii: 1.75 inches.

Conclusion Questions

Answer in complete sentences.

- Give an example of a mechanism that converts rotary motion into linear motion.

- Why do engineers care about the type of motion a mechanism produces?

- A 24-tooth gear on the motor drives a 60-tooth gear on a wheel. What is the gear ratio, what is the output speed if the motor spins 200 RPM, and which shaft has more torque?

- Fill in the blanks: As the torque of a gear train increases, the speed _______. To increase torque, you should _______ the size of the drive gear or _______ the size of the driven gear.

- Pedaling a bike up a steep hill, you shift to a low gear. Which way are you trading speed and torque, and why does it help?

- Your robot's arm motor whines and quits when lifting a full water bottle. Without a bigger motor, what's the fix — and what will you give up?

Automation & Robotics · VEX EXP · C++ in VS Code — Student Edition