PROJECT 2.3 · UNIT 2 — MECHANISMS & MOTION Windmill Construction Student Edition — VEX EXP · C++ in VS Code

Name: ______________________________     Date: ____________     Period: ________

Introduction

Your first real engineering project — a problem to solve, not a worksheet with an answer key. A windmill captures rotary motion and turns it into useful work; your job is to build a geared windmill that lifts a small load. You'll run the full engineering design process — define, brainstorm, select with a decision matrix, build, test, and improve — and put Unit 2 to work: a bevel gear to turn the corner 90° and a gear ratio chosen for enough torque to lift. Document every step in your engineering notebook. Read Chapter 5 in the textbook for the full explanation.

Learning Objectives

- Apply the engineering design process from problem definition through iteration.

- Use a decision matrix to choose among design ideas on purpose, not by gut feeling.

- Use a bevel gear to change the direction of motion 90°, and select/justify a gear ratio for the lifting task.

- Document the process thoroughly in an engineering notebook.

Key Ideas (quick reference)

- The design loop:  define → brainstorm (≥3 sketches) → select (decision matrix) → build → test → improve, then loop. Documenting in the notebook runs through every step. Cycling Test → Improve → Build several times is normal.

- Decision matrix:  list criteria, weight each 1–5 by importance, score each concept 1–5, then score × weight and total the columns. The highest total is your starting design — and the math shows which features were worth it.

- Turn the corner:  a bevel gear transfers rotation 'around the corner,' changing the axis of motion 90° — the windmill's signature mechanism.

- Gear for lift:  lifting takes torque. If it spins free but stalls under load, gear DOWN (slower drum, more pull). The trade is speed — for lifting, slower-but-stronger wins.

Build Constraints

Build constraints (all year): (1) Build from the EXP kit — including bevel gears from the Advanced Gear Kit; these are shared with V5, so build from the EXP supply to keep the competition kits complete. (2) The whole windmill must fit one storage cubby, about 33 × 33 × 39 cm (13 × 13 × 15 in). Sketch with that envelope in mind, and break the build down to store it.

Safety

• Keep fingers, hair, and loose clothing clear of meshing gears and the spinning rotor — gear teeth pinch. • The lifted load can swing or drop: keep it light, lift it over the table (not over hands or laps), and stand clear of the drop zone. • If a motor drives the rotor, clear the gear train before it spins and stop it before adjusting anything.

Equipment & Materials

- VEX EXP kit — bevel gears (Advanced Gear Kit) and spur gears (24/36/48/60T)

- A gearbox bracket to hold the bevel pair in proper 90° mesh

- Shafts, bearing flats, shaft collars, C-channels/plates for a rigid frame

- String and a small drum/spool; the standard load your teacher provides

- Design brief and decision-matrix templates (below); engineering notebook

- Optional: an EXP Smart Motor or hand crank to drive the rotor; a fan for true 'wind'

Procedure

- Define.  Read the design brief and restate the problem and constraints in your notebook, in your own words.

- Brainstorm.  Sketch at least three windmill concepts. Label the input (rotor) and output (lifting drum) on each, and note how each turns the corner 90° and where its gearing goes.

- Select.  Fill in the decision matrix below (criteria, weights, scores), total the columns, and choose your design. Record the choice.

- Build.  Construct your windmill from EXP parts. Keep the frame rigid so the gears stay meshed, and confirm it fits the cubby.

- Model the three gearings.  Set up A (1:1), then B (geared down for torque), then C (geared up for speed). Name the mechanism in each and record what happens to the lift.

- Test & improve.  Try to lift the load. If it stalls, gear down further or stiffen the structure; loop until it lifts reliably, recording each change.

- Gallery walk.  Compare with other teams' windmills; give and collect peer feedback in your notebook.

Design Brief — restate it in your own words

| Design brief | Your restatement |
| --- | --- |
| Client | Your teacher (someone who needs a load lifted). |
| Problem statement |  |
| Design statement |  |
| Constraints | EXP parts only · fits the cubby (≈33×33×39 cm) · changes direction 90° · stable & rigid |
| Deliverables | Working windmill · completed decision matrix · documented notebook |

Decision Matrix

Weight each criterion 1–5, score each concept 1–5, then write score → score × weight. Total the columns; the highest total is your starting design.

| Criterion | Weight (1–5) | Concept A (score → ×wt) | Concept B | Concept C |
| --- | --- | --- | --- | --- |
| Lifting power (torque) |  |  |  |  |
| Turns the corner 90° |  |  |  |  |
| Stable & rigid |  |  |  |  |
| Easy to build |  |  |  |  |
| Fits the cubby |  |  |  |  |
| Total |  |  |  |  |

Engineering Drawing

A VEX beam, dimensioned. Holes are one pitch (0.50 in) apart, center-to-center — count the spaces to get the length, and to check the cubby fit.

Conclusion Questions

Answer in complete sentences.

- How did your gear ratio choice affect the windmill's ability to lift the load?

- What would you change if you built it again, and why?

- Which mechanisms from Unit 2 can increase torque and decrease speed? (List all that apply.)

- Which mechanisms can transmit rotary motion at a 90° angle?

- What is the purpose of an idler gear — and did your windmill use one?

- How did the decision matrix change (or confirm) the design you would have picked by gut feeling?

Automation & Robotics · VEX EXP · C++ in VS Code — Student Edition