ACTIVITY 2.2 · UNIT 2 — MECHANISMS & MOTION Mechanical Gears Student Edition — VEX EXP · C++ in VS Code

Name: ______________________________     Date: ____________     Period: ________

Introduction

Gears trade speed for torque — and this chapter pushes that idea three steps further. An idler gear placed between the drive and driven gears changes the direction but not the ratio — only the first and last gears count. To reach a big ratio, you compound: two gears share a shaft so the stages multiply (two 2.5:1 stages give 6.25:1). And not every gear is a flat spur gear — your kit also has bevel gears (turn a 90° corner), a worm & wheel (huge one-step reduction that won't back-drive), rack & pinion (rotary becomes straight-line motion), and sprockets & chain (send power across a distance). In this activity you'll build gear trains, verify ratios by counting turns, and chart what each mechanism can do. Read Chapter 4 in the textbook for the full explanation.

Learning Objectives

- Calculate a gear ratio from tooth counts and predict whether a train gears up or gears down.

- Explain why an idler gear changes direction but not the ratio.

- Build a compound gear train and find its overall ratio by multiplying the stages.

- Identify the five gear/sprocket types in the EXP kit and state what each is for.

- Verify a predicted ratio by counting turns of the drive and driven shafts.

Key Ideas (quick reference)

- Idlers don't change the ratio.  Only the first (drive) and last (driven) gears set the ratio; any gears between just pass motion along and cancel out. Use an idler to bridge a gap or flip direction. Odd # of gears in a line → same direction; even → opposite.

- Compound = multiply the stages.  When two gears share a locked shaft, the middle counts DON'T cancel. Total ratio = stage 1 × stage 2. Two 2.5:1 stages → 6.25:1. This is how you beat the kit's 2.5:1 single-stage limit.

- Five types, five jobs.  Spur (everyday, one plane) · Sprocket & chain (power across a distance, same direction) · Bevel (90° corner) · Worm & wheel (huge reduction, self-locks, won't back-drive) · Rack & pinion (rotary → linear).

- The worm's superpower.  A single-start worm turns a 24-tooth wheel one tooth per turn = 24:1 from one pair, and the wheel can't drive the worm back — so a worm-driven lift holds its height with the motor off.

Build Constraints

Build constraints (all year): (1) Build from your classroom Gateway kit. It's built around EXP but also bundles some V5 mechanism parts (the Smart Motor, sensors, the Winch & Pulley Kit) — those are yours to use. What's off-limits is the separate competition team's V5 kit: don't pull from it for class builds, so it stays complete for the team. (2) Your whole build must fit one storage cubby — an IKEA Kallax shelf compartment: a 33 × 33 cm (13 × 13 in) opening, about 38 cm (15 in) deep. The 33 × 33 cm face is the binding limit (the build must pass through it). Design compact and break builds down to store them.

Equipment & Materials

- VEX EXP gears (24T, 36T, 48T, 60T), at least one worm + worm wheel, bevel gears, a rack + pinion, sprockets + chain

- Shafts, bearing flats, shaft collars, spacers, and a C-channel or flat-bar baseplate

- A hand crank (or a free shaft to turn by hand); tape to mark a tooth

- Engineering notebook; optional: one EXP Smart Motor to drive a train

Procedure

- Inventory your gears.  Lay out the 24/36/48/60T spur gears and count teeth to confirm the numbers.

- Build a simple pair and verify it.  Mesh a small drive gear with a larger driven gear. Tape-mark a tooth on each, turn the drive exactly 5 full turns, and count the driven turns. Does turns-in ÷ turns-out match the ratio from tooth counts?

- Add an idler — watch it not matter.  Insert a third gear between them. Predict the new ratio (trick question), then re-run the 5-turn test. Note the new output direction.

- Compound it.  Build a two-stage train with a shared middle shaft. Predict the overall ratio by multiplying the stages, then measure by counting turns. Record predicted vs. measured.

- Meet the special gears.  Build or examine a worm & wheel (try to back-drive it — you can't), a rack & pinion (rotary → linear), and a bevel pair (power turns a 90° corner).

- Fill in the mechanism chart  below for each mechanism you built.

Compound Gear-Train Practice

Each stage = driven : drive. Overall ratio = stage 1 × stage 2. First row is worked for you.

| Stage 1 (drive→driven) | Stage 2 (drive→driven) | Overall ratio | Speed & torque effect |
| --- | --- | --- | --- |
| 24T → 48T (2:1) | 24T → 60T (2.5:1) | 2 × 2.5 = 5:1 | Output 5× slower, ~5× torque. Stages needn't match — multiply them. |
| 24T → 60T | 24T → 60T |  |  |
| 36T → 60T | 24T → 48T |  |  |
| single 24T → 60T | — (none) |  |  |

Mechanism Chart

For each mechanism, mark Y or N. The first row is filled in as a model.

| Mechanism | ↑ Speed? | ↑ Torque? | Reversible? | Changes motion type? | Real-life example |
| --- | --- | --- | --- | --- | --- |
| Simple gear train | Y | Y | Y | N | Drivetrains, clocks |
| Simple gear train with idler |  |  |  |  |  |
| Bevel gear |  |  |  |  |  |
| Worm & wheel |  |  |  |  |  |
| Rack & pinion |  |  |  |  |  |
| Sprocket & chain |  |  |  |  |  |
| Belt & pulley |  |  |  |  |  |

Engineering Drawing

A bevel gear pair turns the axis of motion 90° — the windmill's key mechanism in the next chapter.

Code It — One Line, Timed

In Chapter 3 you started and stopped a motor by hand. spinFor does a timed run in one line — the motor stops on its own. In the starter's marked region, type:

Type this in the marked region

leftMotor.setVelocity(50, percent); leftMotor.spinFor(forward, 3, seconds);

Now try:  build a compound train (the multiply rule) and run it for 3 seconds. A big reduction barely creeps — but with far more force; geared up it whirls. Change 3 to 6 to go twice as far. The compounded ratio, not the code, sets how far the output turns.

Conclusion Questions

Answer in complete sentences.

- A 12-tooth driver turns a 36-tooth driven gear. What is the gear ratio, and does this gear up or down?

- You need a robot arm to lift a heavy load slowly and steadily. Gear up or gear down?

- A 24T gear drives a 48T idler, which drives a 60T gear. What is the overall ratio — and what did the idler change?

- Build a compound train from your kit's gears that reaches a reduction of at least 6:1. List the gears in each stage and show the multiplication.

- Your robot's arm needs to hold its position when the motor is off, or it sags. Which gear type solves this, and why?

- You want a claw that opens and closes in a straight line. Which mechanism converts the motor's rotary motion into that linear motion?

Automation & Robotics · VEX EXP · C++ in VS Code — Student Edition