ACTIVITY 1.1 · UNIT 1 — BUILDING THE BODY OF A ROBOT Structures & Building Teacher Copy — VEX EXP · C++ in VS Code

Pacing & prep. Activity 1.1, about Weeks 2–3 on the block schedule (sync to your pacing calendar). No electronics this lesson, so no charging needed. Each station: a sorted kit, T15 key + open-end wrench. Set out identical test loads, and decide the load cap before class and write it on the board.

Introduction

Before a robot can move, sense, or run a program, it needs a frame — the chassis — to hold everything together. If the frame flexes, the wheels, arms, and motors mounted to it flex too. In this activity you'll meet the EXP building system and its two tools, learn why triangulation makes a structure rigid, then build a structure and brace it. Read Chapter 1 in the textbook for the full explanation.

Learning Objectives

- Identify the main VEX EXP structural parts (C-channels, flat bars, plates, gussets, standoffs) and the two kit tools.

- Explain why triangulation makes a structure rigid, and why a single-screw joint acts as a hinge.

- Build a stable structure and safely load-test it for flex, then brace it to reduce wobble.

- Practice organized, safe use of the kit and a parts-management routine.

Key Ideas (quick reference)

Parts: C-channels (strongest), flat bars, plates (for stiffening, not main structure), gussets (brace corners), standoffs, shafts/collars/bearings. Tools: T15 Star Drive Key turns the #8-32 Star Drive Screw; the open-end wrench holds the #8-32 Low Profile Nut. Wrench holds the nut, star key turns the screw. One screw is a hinge: a single-screw joint pivots — use two screws to lock an angle. Triangulation: a diagonal turns a wobbly square into locked triangles. When you can't add a diagonal, a 45° gusset does the same job.

Equipment & Materials

- VEX EXP kit (structural metal, standoffs, shafts, fasteners)

- T15 Star Drive Key and open-end wrench

- A standard test load (identical water bottles, books, or a bucket + marbles)

- Engineering notebook

Procedure

- Inventory and sort the kit; set up the parts-management routine you'll keep all semester.

- Practice the joins: bolt a C-channel to a plate with a star screw and Low Profile Nut, using two screws so the joint can't pivot. Mount a shaft so it spins freely but is locked by a collar.

- Build a freestanding tower or bridge of your own design (or one your teacher provides), then load-test it safely — note where it flexes most.

- Brace it. Add a diagonal, a gusset, a standoff, or close a box where it wobbled. Re-test with the same load and record the before/after in your notebook.

Load-Test Safely

Test for flex (wobble that springs back), not failure (permanent bending). EXP metal is aluminum and can take a permanent bend if forced. - Set a load cap before you start and write it down. The test ends at the cap — that's not failure. - No pushing down by hand, standing on it, or levering. Force comes only from the agreed weights or a light finger-push. - Measure deflection; “pass” means it springs back. Reward the stiffest design, not a survived catastrophe.

Conclusion Questions

Answer in complete sentences.

- Using triangulation, explain why a diagonal brace or a 45° gusset makes a structure stronger.

Answer: A diagonal (or gusset) splits a rack-prone square into triangles. A triangle can't change shape unless a side changes length, and the metal won't stretch — so the shape is locked.

- A C-channel and a flat bar are the same length. Which resists bending better, and what about its shape explains that?

Answer: The C-channel. Its folded U-shape spreads material away from the bending axis (the flanges), which resists bending far more than a flat bar of the same length.

- You join two beams with one screw and they keep rotating. What's happening, and what's the fix?

Answer: One screw acts as a hinge, so the parts pivot around it. The fix: add a second screw through another hole to lock the angle.

- Why does adding more screws to a joint make it stronger but isn't always the best choice?

Answer: More screws add connection points and stiffness, but each adds weight. Use enough where forces are high, fewer where they aren't — a strength-vs-weight tradeoff.

- Name one way to stiffen a drivetrain other than adding a diagonal brace.

Answer: Any of: box construction (plate between two C-channels), standoffs as braces, use a C-channel instead of a flat bar, add gussets, or more fasteners at the junction.

- In a load test, why do we measure how far the structure flexes instead of how much weight makes it break?

Answer: The goal is to compare stiffness before/after bracing, not to break parts. Measuring deflection is non-destructive, keeps the aluminum from taking a permanent bend, and gives comparable data.

Teaching Notes

Common misconceptions: “more screws is always better” (weight cost); “plates are structural” (they're for stiffening/mounting); single-screw joints (they hinge). Safety: enforce the three load-test rules; no standing on or pushing down on builds; if a part bends, treat it as a teachable moment — straighten, add a gusset, re-test. Extension: students new to building can follow VEX's BaseBot guide; challenge fast finishers to pass the same load with the fewest parts.

Automation & Robotics · VEX EXP · C++ in VS Code — Teacher Copy